100920 HP25C Celestial Triangle Reduction

Angles in R1 → R5 stored in °   '   "

Azimuth in R6 stored in decimal degrees

Registers Allocation :

REG 1 : D , Body declination ( °  '  " )

REG 2 : T , Local Hour angle ( °  '  " )

REG 3 : Hv, Observed Geocentric Height ( °  '  " )

REG 4 : Lat , Observer's Latitude ( °  '  " )

REG 5 : Ho, Computed Geocentric Height ( ° '  " )

REG 6 : He, Computed Azimuth ( ° )

This is the program :

1 - f FIX 1

2 - RCL 1

3 - g→H

4 - 1

5 - f→R

6 - RCL 2

7 - g→H

8 - 9

9 - 0

10 - +

11 - x<>y

12 - f→R

13 - ENTER

14 - RDN (roll the stack down, shows as "R with down arrow" on the HP25C calculator)

15 - RDN

16 - g→P

17 - x<>y

18 - RCL 4

19 - g→H

20 -  - ("minus" sign)

21 - x<>y

22 - f→R

23 - RDN

24 - x<>y

25 - RDN

26 - g→P

27 - x<>y

28 - gx>=0

29 - GTO34

30 - 3

31 - 6

32 - 0

33 - +

34 - STO 6

35 - RDN

36 - g→P

37 - x<>y

38 - f→HMS

39 - STO 5

40 - f LASTx

41 - RCL 3

42 - g→H

43 - x<>y

44 - - (minus sign)

45 - 6

46 - 0

47 - *

48 - R/S (displays the Intercept in NM)

49 - RCL 6 (displays the Azimut in °)

To run the Program :

Calculator must be set in Degrees (g DEG), then enter the following :

Declination D ( °  '  " ) STO 1,

Hour Angle  T ( °  '  " ) STO 2,

Observed geocentric height Hv ( °  '  " ) STO 3

Observer's Latitude Lat ( °  '  " ) STO 4

GTO 00 R/S

Step 48 : The program will first halt displaying the intercept value in NM ( "Observed – Computed" geocentric heights, i.e. the value of "Hv-He" in arc minutes),

then R/S :

Step 49 : The program will end and show the computed Azimuth reckoned from true north ( 0° - 360° range)

EXAMPLE 1 :

R 1 : D     =  S  -13°25'48"

R 2 : T     =        40°53'18"

R 3 : Hv   =       50°14'18"

R 4 : Lat =  S  -14°02'54", get :

Intercept = -5.3 NM (i.e. AWAY from body),

R 5 : He   =   50°19'39" , and

R 6 : Z     =      265°8

EXAMPLE 2 :

R 1 : D= N +34°31'12"

R 2 : T=36°18'00

R 3 : Hv=60°35'14"

R 4 : Lat=N37°35'24", get :

Intercept = -5.3 NM (i.e. AWAY from body),

R 5 : He=60°40'22" , and

R 6 : Z = 275°2

Antoine M. Couëtte

Cellphone : +33-(0)6 12 88 06 36

antoine .m.couette@club-internet.fr