| Table 1 Dip Angles Measured with Theodolite (1) – Minutes of Arc by B.J.Pennino |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test No. | Location Date | Time (2) | Hgt. of Eye- ft | Dip Angle ‘ | KM (3) | TA F | TW F | Atmos Pr. in-Hg | Comments & Conditions |
| 1,2,3 | Prel. tests | only ; | no | dip | data. |  |  |  |  |
| 4A | BH 4/1/13 | 1100 | 676 +/- | 24.5 | 0.94 | 55 | 42 | 29.56 | Mostly cloudy, wind @ |
| 4B | BH 4/1/13 | 1300 | 627+/- | 23.1 | 0.92 | 55 | 42 | 29.56 | 12mph |
| 5A | APS 4/4/13 | 1233 | 13.5 | 4.4 | 1.20 | 34 | 42 |  | Sight to 240 M, SW, SU (5) |
| 5B | APMSD “ | 1730 | 13.1 | 3.0 | 0.83 | 34 | 42 | 30.31 | Sight to Race, C (6) |
| 5C | APLH 4/4/13 | 1815 | 22.5+/- | 4.4 | 0.93 | 34 | 42 | 30.23 | Sight Race, SW, MW, C |
| 6A | NHB 4/14/13 | 1030 | 33.5 | 4.1 | 0.71 | 48 | 43 | 30.10 | C, LW, SU, A |
| 6B | NHB 4/14/13 | 1100 | 32.4 | 4.3 | 0.74 | 48 | 43 | 30.10 | C,LW @8mph, SU, A |
| 7 | FEB 4/14/13 | 1630 | 22.4 | 3.9 | 0.82 | 48 | 43 | 30.07 | C,SP,W@13 mph, CCB |
| 11A | NHB 5/26/13 | 1400 | 11.7 | 2.4 | 0.70 | 55 | 53 | 29.90 | C,SP, W@12, A |
| 11B | NHB 5/26/13 | 1445 | 22.9 | 3.6 | 0.75 | 55 | 53 | 29.90 | C,CL,W@12, A |
| 8A | NHB 4/15/13 | 0925 | 34.7 | 5.3 | 0.89 | 46 | 43 | 30.40 | C,LW@9, A |
| 8B | NHB 4/15/13 | 0950 | 17.9 | 3.8 | 0.89 | 46 | 43 | 30.40 | “ “ , A |
| 21 | NHB 7/21/13 | 2000 | 28.5+/- | 5.7 | 1.07 | 68 | 68 | 29.98 | C,S,SS, LW@ 6mph, A |
| 23 | NHB 8/14/13 | 1500 | 27.5 | 4.8 | 0.92 | 75 | 67 | 29.77 | SU, W@ 13mph, A |
| 24 | NHB 8/16/13 | 0930 | 29.5+/- | 4.5 | 0.83 | 72 | 66 | 30.16 | SU, C, W@12 mph, A |
| 25 | NHB 8/16/13 | 1730 | 28.5+/- | 3.6 | 0.67 | 74 | 66 | 30.16 | SU,C, W@ 15 mph, A |
| 8EP | EP 4/26/13 | 1200 | 13.0 | 2.4 | 0.66 | 57 | 47 | 30.35 | Sight to Race ,C, SU |
| 9A | APS 4/26/13 | 1321 | 11.7+/- | 3.0 | 0.88 | 57 | 47 | 30.35 | Sight to 240 M, C, LW |
| 9B | APS “ “ | 1335 | 11.6 | 2.3 | 0.68 | 57 | 47 | 30.35 | Sight to Race , “ “ |
| 9A’* | APS “ “ | 1341 | 11.6 | 3.3 | 0.97 | 57 | 47 | 30.35 | Note 4 Sight to 240 M |
| 9B’* | APS “ “ | 1342 | 11.6 | 2.4 | 0.70 | 57 | 47 | 30.35 | Sight to Race, C, LW, SU |
| 9C | APMSD “ “ | 1443 | 14.6 | 3.7 | 0.97 | 57 | 47 | 30.35 | Sight to 240 M, C, LW, SU |
| 9C’ | APMSD “ “ | 1456 | 14.6 | 2.8 | 0.73 | 57 | 47 | 30.35 | Sight to Race, C,LW, SU |
| 9D | APLH “ “ | 1518 | 29.2 | 4.6 | 0.85 | 57 | 47 | 30.35 | Sight to 240 M,C, LW, SU |
| 9D’ | APLH “ “ | 1529 | 29.2 | 4.2 | 0.78 | 57 | 47 | 30.35 | Sight to Race, C, LW, SU |
| 10A | MSPL 4/27/13 | 1615 | 116+/- | 10.0 | 0.93 | 56 | 47 | 30.36 | C, NW , SU, A |
| 10B | MSPV 4/27/13 | 1720 | 102+/- | 9.2 | 0.91 | 56 | 47 | 30.36 | C, NW , SU, A |
| 12A | MSPL 5/26/13 | 1533 | 112+/- | 9.4 | 0.89 | 55 | 53 | 29.90 | C,CL,W@12, A |
| 12B | MSPL 5/26/13 | 1602 | 113+/- | 9.5 | 0.89 | 55 | 53 | 29.90 | C,CL,W@12, A |
| 12C | MSPV 5/26/13 | 1545 | 99+/- | 9.0 | 0.90 | 55 | 53 | 29.90 | C,CL,W@12, A |
| !3A | HLOD 5/27/13 | 1037 | 144+/- | 10.5 | 0.88 | 65 | 54 | 30.11 | Sight to SE, SU,C,LW, A |
| 13C | HLOD 5/26/13 | 1100 | 143+/- | 10.8 | 0.90 | 65 | 54 | 30.11 | Sight to NNE, SU,C,LW, A |
| 15 | HLOD 6/10/13 | 1345 | 139 +/- | 10.8 | 0.92 | 65 | 55 | 30.21 | C,LW, VCL, HF, A |
| 14 | LEHB 6/10/13 | 1245 | 43.8 | 6.3 | 0.98 | 65 | 55 | 30.21 | C, SU, LW, A |
| 16 | MSPL 7/03/13 | 1947 | 108 +/- | 8.4 | 0.81 | 74 | 61 | 30.18 | Wavy Hor., SU,W@15, A |
| 17 | DuHB 7/06/13 | 2030 | 19.5 | 3.4 | 0.77 | 75 | 62 | 30.00 | C, SS,HF W@ 5mph,CCB |
| 19 | CHB 7/14//13 | 2000 | 19 +/- | 3.3 | 0.76 | 78 | 65 | 29.93 | C, SS, CCB |
| 20 | CHB 7/15/13 | 1830 | 14 +/- | 2.8 | 0.75 | 80 | 67 | 30.12 | C, NW, SU, CCB |
| 22* | CHB 7/27/13 | 2030 | 20.5+/- | 2.8 | 0.66 | 71 | 69 | 30.07 | C, SU W@ 7 mph, SS, CCB |
| 26 | CHB 8/17/13 | 2000 | 11.4 | 4.9 | 1.45 | 63 | 66 | 30.27 | C, SS,W@5mph,CCB |
| 27 | CHB 8/20/13 | 1958 | 18.5 | 3.8 | 0.87 | 75 | 66 | 30.07 | C, SS, W@8 mph, CCB |
| 28 | BVTL 9/28/13 | 1300 | 29.6 | 5.2 | 0.96 | 64 | 65 | 30.27 | C, SU,W@6mph, A |
| 29 | NHB 11/03/13 | 1500 | 33 +/- | 6.3 | 1.1 | 45 | 56 | 30.13 | WC,SU,SW@15++mph ,A |

ALL DATA   Mean Value   KM = 0.87

Notes: 1. Topcon EDM/ Theodolite GTS – 2B;   2. Time- Local, USA, EDT or EST; 3. KM = Measured dip angle / sqrtHoE;

4. Dip angle usually avg. of 10 readings; * data- avg. of 4 readings; 5. Sight to 240 M Az is down Long Island Sound to south west @ mag. compass az.. 6. Sight to Race to north east entrance to Long Island Sound (LIS)

C – Calm sea surface, WC- white caps;  NW- No Wind; LW- Light Wind; MW-Moderate Wind; W@ (mph); SW – Strong wind;   SU-Sunny, SP-Partly Sunny; SS- near sunset; CL – Cloudy; VCL – Very cloudy/overcast/hazy;  HF or HP- Horiz. Faint or Poor

Locations: A – Sight Atlantic Horizon;  APS – Avery Point Sail boat launch &  storage area (parking lot), UConn. campus; APMSD -  Avery Point behind Marine Science UConn Bldg., on dock;  APLH- Avery Point Lighthouse (near); BH- Blue Hill Observatory;   BVTL – Beavertail  Light Jamestown, RI ;  CCB –  Sight Cape Cod Bay Horizon;  CHB - Corn Hill Beach Truro, MA; DuHB - Duck Harbor Beach, Wellfleet, MA;  EP- Eastern Point, Groton, CT; FEB- First  Encoun. Beach, Eastham, MA.  HLOD - Highland Light Observ. Deck;  N. Truro, MA; LEHB – Lecount Hollow Beach ;  MSPL - Marconi Station Observ. Platform; MSPV – Marconi Stat. Pavilion;   NHB – Newcomb Hollow Beach, Wellfleet,MA

| Table 2 Accuracy - Ho - Hc (measurements by theodolite, see table 1 data and information) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # Date UT Location | Celestial Body | Time UT Hr:mn:sec | Hgt. of Eye ft | Meas. Dip ‘ Dm (Km) | NA Dip Min (1) | Zenith Angle Deg-mn-sec | Hs 90 - Za Deg-min | Ho Deg -min | Hc Deg- min | Ho - Hc Deg-min |
| # 17 7/07/13 DuHB | Venus | 00:44:47 | 19.5 | Note 2 3.4 (0.78) | 4.3 | 79 - 22 - 00 | 10d 38.0’ | 10d 33.1’ | 10d 33.1’ | 0.0 |
| # 17 7/07/13 DuHB | Venus | 00:50:11 | 19.0 | 3.4 (0.78)* | 4.3 | 79 - 47- 54 | 10d 12.1’ | 10d 07.0’ | 10d 7.3’ | - 0.3 |
| # 19 7/15/13 CHB | Venus | 00:28:36 | 19.5 | 3.3 (0.78) | 4.3 | 76 - 30 - 02 | 13d 30.0’ | 13d 26.0’ | 13d 26.0’ | 0.0 |
| # 21 7/22/13 NHB | Venus | 00:37:04 | 28.5 | 5.7 (1.08) | 5.2 | 76 - 51- 21 | 13d 08.7’ | 14d 20.9’ | 14d 20.7’ | 0.2 |
| # 22 7/28/13 CHB | Moon LL | 00:30:35 | 20.5 | 3.0 (0.66) | 4.4 | 78 - 55 - 36 | 11d 04.4’ | 10d 59.7’ | 11d 00.0’ | - 0.3 |
| # 23 8/14/13 NHB | Moon UL | 19:49:05 | 27.5 | 4.8 (0.91) | 5.1 | 73 - 18 - 55 | 16d 41.1’ | 17d 18.2’ | 17d 8.1’ | 0.1 |
| # 25 8/16/13 NHB | Moon UL | 21:24:47 | 28.5 | 3.6 (0.67) | 5.2 | 77 - 28 - 06 | 12d 31.9’ | 13d 09.8’ | 13d 09.9’ | - 0.1 |
| # 27 8/20/13 CHB | Venus | 23:59:57 | 18.5 | 3.8 (0.87) | 4.2 | 79 - 30 - 38 | 10d 29.4 | 10d 24.3’ | 10d 24.3’ | 0.0 |
| # 28 9/28/13 BVTL | Moon UL | 16:37:10 | 29.6 | 5.2 (0.96) | 5.3 | 66 - 24 - 06 | 23d 35.9 | 24d 08.6’ | 24d 08.4’ | 0.2 |

This table  -   Mean  Value  KM  = 0.84

Table 1 and 2 Data Measured and

* Not included in mean because same date as earlier sight.                                  Analyzed by Bruce J. Pennino, P.E.

Holden, MA 01520 USA

Notes:

1. NA = Nautical Almanac dip table where  dip (minutes) = 0.97 sqrt Height of eye (ft)

2. KM  is defined herein as dip coefficient calculated  from  theodolite measured shown dip (Table 1) and height of eye .

KM= dip / sqrtHoE (ft)

3. HC = calculated altitude of sight of celestial body at that known  location  with

Sin HC = Sin(Lat)xSin(Dec) + Cos(Lat)xCos(Dec)xCos(LHA)   where

Lat = Latitude of location, Dec = Declination of body sighted , and LHA =  Local hour angle