130612 NAVLIST Moon Parallax Comparison between Accurate 3D Space formulae and approximate Nautical Almanac Formulae.

Date Sep 29th, 1987

TT-UT=55.7 S

All MOON observations assumed to be carried out at the very same time (UT=23H48M29.0S) from different places specified further down.

Same environment everywhere: HOE=0ft, T=10°C, P=1013.25mb

********

EXAMPLE #1

From Position:

S 90°00’0 Longitude: any

At UT=23h48m29.0s

Sextant Height=27°37’.242832

MOON I=0.0 NM Z=any

3D Results

DIP=0.000 MIN

RUN

REF=-1.844 MIN

RUN

SD AUG.=16.159 MIN

RUN

***PAR.=51.831 MIN***

RUN

SDA+P= 67.990 MIN

RUN

R+SD+P=66.146 MIN

RUN

GEOC.H=28.43389 D.MX

NAL Formulae Results

DIP=0.000 MIN

RUN

REF=-1.844 MIN

RUN

SD AUG.=16.157 MIN

RUN

***PAR.=51.836 MIN***

RUN

SDA+P= 67.992 MIN

RUN

R+SD+P=66.149 MIN

RUN

GEOC.H=28.43391 D.MX

Difference in Parallax between Formulae: 0”30

*******

The results showing under green color are 3D space results, i.e. fully accurate ones.

The results showing under purple color are results from a straight/direct (i.e no Loop) 2D space computation - which already yields Augmented Semi-Diameters accurate to better than +/- 0”3 and Parallaxes accurate to +/- 0’2 – with its Parallaxes results further corrected with the following Formulae published in the NAL Explanation Part as follows:

PA=HP cosH + OB, with

OB= -0°.0032 sin²Lat cosH

+ 0°0032 sin(2Lat) cosZ sinH

EXAMPLE #2

From Position:

S 24°04’6 W094°52’6

At UT=23h48m29.0s

Sextant Height=85°00’.0

MOON I=0.0 NM Z=180.0°

3D Results

DIP=0.000 MIN

RUN

REF=-0.085 MIN

RUN

SD AUG.=16.306 MIN

RUN

***PAR.=4.994 MIN***

RUN

SDA+P= 21.300 MIN

RUN

R+SD+P=21.215 MIN

RUN

GEOC.H=85.21215 D.MX

NAL Formulae Results

DIP=0.000 MIN

RUN

REF=-0.085 MIN

RUN

SD AUG.=16.302 MIN

RUN

***PAR.=4.990 MIN***

RUN

SDA+P= 21.292 MIN

RUN

R+SD+P=21.207 MIN

RUN

GEOC.H=85.21207 D.MX

Difference in Parallax between Formulae: 0”24

*******

EXAMPLE #3

From Position:

S 28°43’4 W089°34’5

At UT=23h48m29.0s

Sextant Height=85°00’.0

MOON I=0.0 NM Z=268.7°

3D Results

DIP=0.000 MIN

RUN

REF=-0.085 MIN

RUN

SD AUG.=16.306 MIN

RUN

***PAR.=4.850 MIN***

RUN

SDA+P= 21.156 MIN

RUN

R+SD+P=21.072 MIN

RUN

GEOC.H=85.21072 D.MX

NAL Formulae results

DIP=0.000 MIN

RUN

REF=-0.085 MIN

RUN

SD AUG.=16.302 MIN

RUN

***PAR.=4.850 MIN***

RUN

SDA+P= 21.152 MIN

RUN

R+SD+P=21.067 MIN

RUN

GEOC.H=85.21067 D.MX

Difference in Parallax between Formulae: 0”00

*******

EXAMPLE #4

From Position:

S 33°22’5 W094°52’6

At UT=23h48m29.0s

Sextant Height=85°00’.0

MOON I=0.0 NM Z=000.0°

3D Results

DIP=0.000 MIN

RUN

REF=-0.085 MIN

RUN

SD AUG.=16.306 MIN

RUN

***PAR.=4.665 MIN***

RUN

SDA+P= 20.971 MIN

RUN

R+SD+P=20.886 MIN

RUN

GEOC.H=85.20886 D.MX

NAL Formulae results

DIP=0.000 MIN

RUN

REF=-0.085 MIN

RUN

SD AUG.=16.302 MIN

RUN

***PAR.=4.670 MIN***

RUN

SDA+P= 20.971 MIN

RUN

R+SD+P=20.886 MIN

RUN

GEOC.H=85.20886 D.MX

Difference in Parallax between Formulae: 0”30

*******

Antoine M. “Kermit” Couëtte